# Honda CAN-FD Radar Replacement — Learnings & Status

Car: ACURA_MDX_4G_MMR (and other Honda Bosch CAN-FD: PILOT_4G, PASSPORT_4G, ACCORD_11G, CRV_6G, MDX_4G_MMR).

opendbc repo: mvl-boston/opendbc.  openpilot branch used on-car: mdx-25-alphalong.  Context date: 2026-07-23.

This document was produced by analyzing drive logs (via the comma API) and CI test_models runs. Open it in a session with repo/PR access to continue.

# TL;DR — Current Status

GOAL: On CAN-FD Bosch Honda, openpilot disables the stock radar and impersonates it (sends "radar look-alike" CAN messages) so the camera/ADAS keeps working while openpilot does longitudinal control.

WORKING: Radar look-alike message content, checksums/counters, dual-bus delivery, and now MESSAGE TIMING all match the stock radar. The prior card-process crash (gasfactor AttributeError) is fixed.

STILL FAULTING: The camera sets RDM_PROBLEM (radar module) + LKAS_PROBLEM at cutover and latches them. Root cause is NOT data/timing — the camera physically witnesses openpilot disabling the radar because the disable happens ~0.3 s BEFORE the comma relay opens. This needs an openpilot-side sequencing fix (disable radar only after relay-open), which cannot be fixed from opendbc alone.

SECONDARY: cruiseMismatch fires repeatedly (172x, first ~15.7 s) during longitudinal control — separate long-control/engagement issue, not radar-related (carState.accFaulted stayed 0).

# CAN Topology / Relay (critical mental model)

Bus 0 = powertrain / radar-side ACC-CAN (radar object data destined for powertrain; RADAR_REFERENCE 0x3A1 lives here).

Bus 1 = radar bus in CanBus() terms (CanBus(CP).radar = offset+1). The radar TICK REFERENCE messages (0x710/0x730/0x750) live here.

Bus 2 = camera-side ACC-CAN (camera/ADAS transmits here: CAMERA_MESSAGES, LKAS_HUD, ACC_HUD). CanBus(CP).camera = offset+2.

For CAN-FD: CanBus(CP).pt = 0, .radar = 1, .lkas = 0, .camera = 2.

The comma relay separates the camera (bus 2) from the rest. When CLOSED (startup) bus 0 and bus 2 are physically joined, so the camera sees everything. When OPEN (engaged) the panda bridges them in software.

KEY: safety_fwd_hook only forwards PHYSICALLY-RECEIVED messages across the open relay. openpilot’s OWN transmissions are NOT forwarded. So radar look-alikes must be sent explicitly on the camera bus (2) to reach the camera; sending only on bus 0 never reaches it once the relay is open.

# Chain of fixes made (chronological, all merged unless noted)

These progressively removed camera faults and made the impersonation faithful:

supp_tick: subscribe RADAR_SUPP_TICK_REFERENCE (0x710) so BOSCH_SUPPLEMENTAL_CANFD is actually sent (vl_all does not auto-subscribe like vl).

CarController used CS.supp_tick (not self.supp_tick) — attribute lived on CarState.

MUX cycle: LANE_PATH/HUD_OBJECTS MUX must cycle 1-10, 17-26, 33-42, 49-58. A sequential-if bug skipped the bank-start values 17/33/49; fixed with elif.

RADAR_LEAD.TARGET_SPEED_MAYBE must be 140 (stock constant), not 120.

LANE_PATH/HUD_OBJECTS "no detection" sentinels must match stock byte-for-byte: HUD_OBJECTS empty slot = CAR_TYPE=-1, ROTATION=-128, LONG_DIST=196.9, LAT_DIST=204.7; LANE_PATH offsets use 2047 (0x7FF) invalid sentinel per MUX position.

Dual-bus: the six radar look-alikes must be sent on BOTH bus 0 (powertrain) and bus 2 (camera). Pack ONCE then mirror the identical bytes to both buses so the CANPacker counter/checksum do not double-increment.

Radar disable + DTC clear: on disable, clear the radar’s stored DTCs (UDS 0x14) so stale codes don’t re-fault on a later drive. (A global functional-broadcast clear 0x18DB33F1 was also prototyped on the 0111-op-mdx track.)

Diagnostic-forwarding block (PR #598, on 0111-op-mdx track): block radar UDS responses 0x18DAF1B0 from forwarding bus 0 -> bus 2 so the camera doesn’t observe the disable. NOTE: not present on the canfd-timer branch, and does not help the pre-relay-open window.

Tick-timer redesign (canfd-timer branch): time each look-alike to the stock cadence using persistent tick-reference messages. See next section.

gasfactor crash fix (PR #610, OPEN): __init__ set gasfactor_before_maxgas/windfactor_before_maxgas but update() read/wrote gasfactor_before_gasmax/windfactor_before_gasmax -> AttributeError crashed the card process on CAN-FD long. Renamed init to _before_gasmax.

# Radar timing design (the tick-reference model) — CONFIRMED WORKING

The radar emits low-rate "tick reference" messages that KEEP RUNNING even after its data messages are disabled (they are network-management-ish), so openpilot uses them as a heartbeat to phase its look-alikes to the exact stock cadence.

Tick references and rates (all persist through the op period):

0x710 RADAR_SUPP_TICK_REFERENCE   1 Hz  (period 100 frames @100Hz)  bus 1
0x730 RADAR_HUD_TICK_REFERENCE   10 Hz  (period  10 frames)          bus 1
0x750 RADAR_50HZ_TICK_REFERENCE  50 Hz  (period   2 frames)          bus 1
0x3A1 RADAR_REFERENCE             5 Hz  (period  20 frames)          bus 0 (NOT bus 1)

Critical detail — the 1-frame carstate->carcontroller pipeline delay:

A tick detected in CarState.update() at frame N is only transmitted by CarController.update() at frame N+1 (measured: with on-arrival detection, op TX lands ~10 ms after the tick).

The stock radar sends each data message in the SAME frame as its tick (0 ms offset).

Therefore fire the pulse one frame BEFORE the next tick (counter == period-1); the +1 transmit delay lands the message on the next tick frame, matching stock.

Counter thresholds (in CarState, incrementing per 100 Hz frame; reset to 0 when the tick arrives):

supp_tick        (BOSCH_SUPPLEMENTAL_CANFD): counter == 99   (1 Hz)
hud_tick         (RADAR_HUD_CANFD):          counter == 9    (10 Hz)
radar_50hz_tick  (LANE_PATH/HUD_OBJECTS):    counter == 1    (50 Hz)
radar_5hz_tick   (RADAR_LEAD):               counter == 11   (RADAR_LEAD lags RADAR_REFERENCE ~120 ms, i.e. 12 frames; fire at 11 so +1 delay = ~120 ms)

Verified against route ad9840558640c31d/00000036--a39d1a8b6c (op transmit vs stock, per message):

message            stock offset   op offset
RADAR_HUD_CANFD     0.0 ms         0.0 ms
BOSCH_SUPP          0.0 ms         0.0 ms
RADAR_LEAD        119.9 ms       119.9 ms
LANE_PATH         ~10  ms         ~0  ms (within one frame)

Gotchas fixed in the tick logic: invalid Python identifiers (self.50hz_* cannot start with a digit); RADAR_REFERENCE must be read from cp (bus 0 / powertrain parser) NOT cp_radar (bus 1); name mismatch self.50hz_tick vs CS.radar_50hz_tick.

Parser subscription (CANParser list syntax) for the radar-bus parser:

parsers[Bus.radar] = CANParser(DBC[CP.carFingerprint][Bus.radar], [
    ("RADAR_SUPP_TICK_REFERENCE", 0),
    ("RADAR_HUD_TICK_REFERENCE", 0),
    ("RADAR_50HZ_TICK_REFERENCE", 0),
  ], CanBus(CP).radar)

# The remaining fault: camera RDM_PROBLEM at cutover (PRIMARY OPEN ISSUE)

Symptom: camera LKAS_HUD (0x33D) on bus 2 sets RDM_PROBLEM (bit 21) and LKAS_PROBLEM (bit 12) 0->1 at ~8.2 s and they LATCH for the whole drive. carState.accFaulted stays 0 (so it is a dash/LKAS warning, not an openpilot ACC hard-fault).

Root cause (from UDS trace on bus 2 while the relay is still CLOSED):

7.33s  OP->RADAR  10 03      enter extended diagnostic session   (visible on bus2)
7.34s  RADAR->OP  50 03      session accepted                    (visible on bus2)
7.35s  OP->RADAR  14 ffffff  ClearDiagnosticInformation (all)    (visible on bus2)
7.96s  RADAR->OP  54         clear OK (after 0x78 pending)
7.98s  OP->RADAR  28 83 03   CommunicationControl disableRxAndTx (visible on bus2)
~8.2s  radar silent -> camera sets RDM_PROBLEM (latched)
~8.3s  relay finally OPENS (op radar TX now appears on bus2/bus130)

The radar is silenced ~0.3 s BEFORE the relay opens, so the camera directly observes the whole disable handshake on its own bus.

The diagnostic-forwarding block (0x18DAF1B0 bus0->bus2) does NOT help here: during the relay-closed window bus 0 and bus 2 are physically the same wire, so forwarding rules are irrelevant.

Radar-disable comes from opendbc CarInterface.init() -> disable_ecu(...) with CONTROL_TYPE.DISABLE_RX_DISABLE_TX. But WHEN init() runs relative to the panda relay opening is controlled by openpilot (card / manager / safety-mode transition), not opendbc.

Recommended fix direction (openpilot-side, to pursue in the repo-access session):

Defer the radar-disable UDS sequence until AFTER the relay is confirmed open, so the camera is already isolated on bus 2 and never witnesses the disable. Investigate card.py / CarInterface.init() ordering vs when controlsAllowed / the car safety mode opens the relay.

Alternative/adjunct: impersonate the radar’s diagnostic responses to the camera (answer the camera’s periodic 0x18DAB0F2 ReadDTC 0x19 02 0C on 0x18DAF2B0 with a healthy no-DTC response) so ongoing health-checks pass. (Camera’s own DTC poll was observed ~once per drive with retries.)

Confirm whether RDM_PROBLEM/LKAS_PROBLEM actually block engagement or are only a HUD warning; if only HUD, it may be acceptable short-term.

# Secondary suspect: cruiseMismatch

onroadEvents showed cruiseMismatch x172, first at ~15.7 s (during longitudinal control).

carState.accFaulted = 0 across the whole drive, so it is not the radar fault; likely an op-long engagement / PCM cruise-state disagreement.

Worth a separate investigation: compare carState.cruiseState.enabled/available vs carControl.enabled and the ACC_CONTROL the controller sends.

# Reference: message addresses

RADAR look-alikes openpilot must send (CAN-FD):
  0x310      RADAR_HUD_CANFD           10 Hz
  0x6CD5558  LANE_PATH                 50 Hz (muxed)
  0x6CD5559  HUD_OBJECTS               50 Hz (muxed)
  0xF31AA5C  RADAR_LEAD                 5 Hz
  0xF31AA52  RADAR_LEAD2                5 Hz
  0x1A45AA4E BOSCH_SUPPLEMENTAL_CANFD   1 Hz
Inputs openpilot reads (persist even when radar disabled):
  0x3A1      RADAR_REFERENCE  (bus 0, 5 Hz; COUNTER -> RADAR_LEAD.CNTR_REF)
  0x710/0x730/0x750  tick references (bus 1)
Diagnostics:
  0x18DAB0F1 tester(F1)->radar(B0) requests   0x18DAF1B0 radar->tester responses
  0x18DAB0F2 camera(F2)->radar(B0) requests   0x18DAF2B0 radar->camera responses (op should impersonate)
  0x18DB33F1 29-bit functional broadcast (global DTC clear)
Camera fault bits: LKAS_HUD 0x33D  LKAS_PROBLEM bit12  RDM_PROBLEM bit21 (radarless HUD_OBJECTS is 0x6CD5557; CAN-FD is 0x6CD5559)

# Reference: PRs

PR #609 (MERGED into canfd-timer): radar tick-timer — subscribe 0x730/0x750, fix syntax/bus/phase, gate radar look-alikes + ACC_HUD to op-longitudinal, mirror LANE_PATH/HUD_OBJECTS to camera bus.

PR #610 (OPEN, base canfd-timer, branch cursor/honda-gasfactor-attr-crash-2a41): fix gasfactor/windfactor _before_maxgas vs _before_gasmax AttributeError crash.

Earlier merged work on the 0111-op-mdx track (NOT on canfd-timer): dual-bus mirroring + safety TX allowlist for both buses, radar DTC clear, global functional clear, diagnostic-forwarding block (0x18DAF1B0). Consider whether these need porting to canfd-timer.

# test_models status

The CAN-FD car PASSES test_models after the op-longitudinal gating fixes (LANE_PATH and ACC_HUD were being authored in stock-ACC where the non-long safety list rejects them).

Remaining test_models failures are NON-CAN-FD and pre-existing: Honda mainOn (panda/openpilot ACC-main mismatch) and radarless HUD_OBJECTS 0x6CD5557 canValid (HudObjectTracker subscribes a message some radarless routes lack).